DAYANANDA SAGAR COLLEGE OF

ARTS, SCIENCE AND COMMERCE

BANGALORE – 560111

DEPARTEMENT OF COMPUTER APPLICATIONS

Master of Computer Applications (MCA)

Project On

"Binary Calculator Using Logic Gates"

Submitted by :-

Neha A Kalaburagi (P03CJ25S126049)
Pruthvish S (P03CJ25S126058)
B Tousif Umar (P03CJ25S126009)

Under the Guidance of:

Mrs. Akshata Deepak
Asst. Professor,
Department of Computer Applictaions

# CONTENTS

| 1. | Introduction and Objective | 01 |
| --- | --- | --- |
| 2. | Block Diagram | 02 - 05 |
| 3. | Simulator Details and Implementation Files | 06 - 07 |
| 4. | Implementation | 08 - 10 |
| 5. | Test Cases and Result | 11 - 12 |
| 6. | Conclusion | 13 |
| 7. | Link of Working Project | 14 |

# INTRODUCTION AND OBJECTIVE

### INTRODUCTION

The Binary Calculator Using Logic Gates is a digital logic simulation project that implements a 4-bit Ripple Carry Adder-Subtractor. This project demonstrates how fundamental logic gates specifically XOR, AND, and OR—can be combined to perform arithmetic operations on binary numbers. A key feature of this system is its ability to perform both addition and subtraction using the same core hardware circuit, controlled by a single "Mode" bit.                  The project provides a dual-perspective result, interpreting output bits in both                    Unsigned (0 to 15) and Signed 2's Complement (−8 to +7) formats.

### OBJECTIVE

The primary objectives of this project are:

- Circuit Simulation: To design and simulate a functional 4-bit adder-subtractor circuit using logic gate principles.

- Mode Control: To implement conditional bit inversion via XOR gates, allowing the circuit to switch between addition (Mode = 0) and subtraction (Mode = 1).

- Arithmetic Logic: To demonstrate the application of 2's Complement arithmetic for performing subtraction through addition hardware.

- Error Detection: To implement and explain Carry Out (for unsigned arithmetic) and Overflow Detection (for signed arithmetic).

- Educational Visualization: To provide a step-by-step breakdown of the ripple carry process to enhance the understanding of bit-level computation.

## BLOCK DIAGRAM

The 4-bit Binary Adder-Subtractor consists of four identical stages, each handling one bit of the 4-bit input. The logic is divided into two main layers:  the Control Layer (XOR gates) and the Arithmetic Layer (Full Adders).

A. Mode Control Logic (The XOR Layer)

The top-most part of the circuit is the Mode (M) control line, which is connected to one input of every XOR gate. This layer acts as a "Conditional Inverter."

- When M = 0 (Addition): The XOR gates pass the B inputs (B0, B1, B2, B3) through to the adders without any change.

- When M = 1 (Subtraction): The XOR gates invert all bits of B, creating the 
1's Complement.

B. The Full Adder (FA) Chain

The circuit uses four Full Adders (FA) connected in series. The output carry of one stage becomes the input carry of the next significant stage.

- Initial Carry-in (C0): The Mode bit is also connected to the Carry-in of the first FA. In subtraction (M=1), this adds the required "+1" to the inverted B bits, effectively completing the 2's Complement of the number.

- A and B Inputs: Each FA receives one bit from number A and the modified bit from number B.

- Ripple Propagation: The carry "ripples" from right to left (FA → FA → FA → FA), which is why the final result (S0, S1, S2, S3) is only stable once the carry has moved through all stages.

C. Output and Status Flags

The bottom and left side of the diagram show the results of the computation:

- Sum Bits (S0 to S3): These represent the 4-bit binary result of the operation.

- Carry-out (Cout): The final carry from the last FA. This indicates if an unsigned addition has exceeded the 4-bit limit (greater than 15).

- Overflow Logic (V): Though simplified in the diagram, the circuit monitors the carry entering the last FA and the carry exiting it. If they are different                                             (Cin ≠ Cout for the last stage), the Overflow flag is triggered, indicating an error in signed arithmetic.

Working Principle: Step-By-Step Calculation

To understand how the logic gates and FA (Full Adder) blocks interact, let’s trace a specific subtraction example: 5 - 3.

Step 1: Input Representation

- Number A (5): 0101

- Number B (3): 0011

- Mode (M): 1 (Subtraction)

Step 2: The XOR Layer (1's Complement)

The Mode bit (M=1) is sent to all XOR gates. Each bit of B is XORed with 1:

- B0: 1 ⊕ 1 = 0

- B1: 1 ⊕ 1 = 0

- B2: 0 ⊕ 1 = 1

- B3: 0 ⊕ 1 = 1

- Resulting B': 1100 (The 1's complement of 3)

Step 3: Completing 2's Complement

The Mode bit (M=1) is also sent to the Initial Carry-in (C0) of the first FA. This adds the required "+1" to the 1's complement:

- B' + C0 = 1100 + 1 = 1101 (This is the 2's complement of 3, which represents -3).

Step 4: The Ripple Carry Process

The four FA blocks now perform a standard addition of A + (-B):

- FA0 (LSB): Adds A0(1) + B'0(0) + C0(1).

  - Sum S0 = 0, Carry C1 = 1.

- FA1: Adds A1(0) + B'1(0) + C1(1).

  - Sum S1 = 1, Carry C2 = 0.

- FA2: Adds A2(1) + B'2(1) + C2(0).

  - Sum S2 = 0, Carry C3 = 1.

- FA3 (MSB): Adds A3(0) + B'3(1) + C3(1).

  - Sum S3 = 0, Carry C4 = 1.

Step 5: Final Result Interpretation

- Binary Output (S3 S2 S1 S0): 0010

- Decimal Result: 2 (Which is correct: 5 - 3 = 2).

- Flags: Cout: 1 (In subtraction, a carry-out of 1 means no "borrow" was needed, and the result is positive).

  - Overflow (V): C3(1) ⊕ C4(1) = 0. Since V=0, the signed result is valid and within the 4-bit range.

## SIMULATOR DETAILS & IMPLEMENTATION FILES

Logic Circuit Simulator (CircuitVerse)

The core gate-level logic of this project was designed, implemented, and verified using CircuitVerse, an online digital logic simulation platform. This environment provided a highly visual and interactive method to validate the mathematical theories of Boolean algebra and 2's complement arithmetic before algorithmic verification in MATLAB.

1. Interactive Hardware Toggles

- Input Bit Array: The simulator features two rows of four interactive switches representing the binary inputs A (A0-A3) and B (B0-B3).

- Manual Manipulation: Users can manually toggle each bit between logic high (1) and logic low (0), allowing for the testing of specific bit patterns and edge cases.

- Binary-to-Decimal Sync: These inputs are directly mapped to the arithmetic layer, providing an immediate visual representation of how binary values are processed by the gates.

2. Mode Control (M) Mechanism

- Operational Switch: A dedicated master toggle, labeled Mode (M), acts as the primary control signal for the entire circuit.

- Logic Engagement: When the Mode switch is set to High (1), the circuit dynamically engages the subtraction logic by activating the XOR inversion layer.

- Arithmetic Shift: When set to Low (0), the circuit performs standard addition, demonstrating how a single hardware layout can be multiplexed to perform different functions.

3. Visual Logic Probes and LEDs

- Real-Time Output: The result of every logic change is instantly visible through glowing LED output probes representing the Sum bits (S0-S3).

- Status Flags: Dedicated probes monitor critical status indicators, including the Carry-out (Cout) for unsigned operations and the Overflow flag (V) for signed 2's complement operations.

- Interpretation: This immediate visual feedback allows for the rapid verification of test cases against expected decimal results.

4. Gate Connectivity and Ripple Propagation

- Structural Transparency: The simulator provides a clear view of the "wiring" between components, specifically showing how the XOR inversion layer connects to the Full Adder (FA) blocks.

- Ripple Visualization: Users can observe the sequential "ripple" effect, where the carry bit generated in the Least Significant Bit (FA0) stage propagates through the intermediate stages (FA1 and FA2) to reach the Most Significant Bit (FA3).

- Educational Insight: This level of connectivity confirms the timing and dependency constraints inherent in a Ripple Carry architecture, illustrating why the final result is only valid once the carry has stabilized through all four stages.

IMPLEMENTATION

The implementation details the logic required to build the 4-bit Ripple Carry Adder-Subtractor hardware.

A. Full Adder Logic

The core of the system is the Full Adder, which adds three bits.

Full Adder Truth Table:

| A | B' | Cin | Sum (S) | Carry Out (Cout) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

Boolean Equations:

- Sum (S):

| A∖BCin​ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 1 | 0 |

- S = A’B’Cin + A’BC’in + AB’C’in + ABCin

- S = A ⨁ B' ⨁ Ci

- Carry Out (Cout):

| A∖BCin​ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 1 | 1 |

- Cout = AB + BCin + ACin

B. Subtractor Logic

Subtraction is achieved through 2's complement addition.

- XOR Conditional Inversion: Input B is processed as B' = B ⨁ Mode. If Mode=1,       B is bitwise inverted (1's complement).

- Initial Carry (C0): The Mode bit is fed into the first carry-in, adding 1 to the                   1's complement to complete the 2's complement.

Full Subtractor Truth Table (for direct subtraction A-B-Bin):

| A | B | Bin | Difference (D) | Borrow-out (Bout) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

Boolean Equations:

- Difference (D):

| A∖BBin​ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 1 | 0 |

- D = A’B’Bin + A’BB’in + AB’B’in + ABBin

- D = A ⨁ B ⨁ Bin

- Borrow-out (Bout):

| A∖BBin​ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 |

- Bout = A’B + A’Bin + BBin

- Bout = AB + (A + B)Bin

C. Signed and Unsigned Logic

- Unsigned: Uses standard binary weighting for a range of 0 to 15.

- Signed (2's Complement): The MSB is treated as a sign bit with a negative weight      (-8), providing a range of −8 to +7.

- Conversion: In both MATLAB and JavaScript, if the MSB (S3) is 1, the signed value is calculated as Unsigned - 16.

D. Overflow Detection

- Logic: Overflow occurs only in signed arithmetic when the result exceeds the range      [-8, +7].

- Implementation: Overflow = C3 ⨁ C4, where C3 is the carry into the MSB and C4 is the carry out.

TEST CASES & RESULTS

The project was validated using a series of strategic test cases designed to verify the accuracy of the ripple carry logic, 2's complement subtraction, and error detection (carry and overflow). The following results were consistent across both the Web Simulator and the MATLAB implementation.

A. Addition Test Cases (Mode = 0)

| Case | Input A | Input B | Binary Result | Unsigned | Signed | Status |
| --- | --- | --- | --- | --- | --- | --- |
| Simple Addition | 0011 (3) | 0010 (2) | 0101 | 5 | 5 | Correct |
| Negative+ Positive | 1001 (-7) | 0110 (6) | 1111 | 15 | -1 | Correct |
| Signed Overflow | 0111 (7) | 0010 (2) | 1001 | 9 | -7 | Overflow Detected |

Observation (Case 3): In signed mode, 7 + 2 = 9. Since 9 is outside the range [-8, +7], the MSB becomes 1, and the circuit detects an overflow (C3 ⨁ C4 = 1).

B. Subtraction Test Cases (Mode = 1)

| Case | Input A | Input B | Binary Result | Unsigned | Signed | Status |
| --- | --- | --- | --- | --- | --- | --- |
| Simple Subtraction | 0111 (7) | 0011 (3) | 0100 | 4 | 4 | Correct |
| Result < 0 (Borrow) | 0011 (3) | 0101 (5) | 1110 | 14 | -2 | Correct |
| Zero Result | 0101 (5) | 0101 (5) | 0000 | 0 | 0 | Correct |

Observation (Case 2): When A < B in unsigned mode, a "borrow" occurs, indicated by     Carry Out = 0. In signed mode, the result 1110 correctly represents -2 via 2's complement.

C. Summary of Results

- Logical Accuracy: The XOR gates correctly inverted Input B during subtraction, and the initial carry-in (C0 =1) successfully completed the 2's complement operation.

- Interpretation: The dual-display system accurately converted binary strings to decimal for both unsigned and signed formats in real-time.

- Flag Verification: The overflow flag correctly triggered only during signed operations where the result exceeded 4-bit boundaries.

Addition (0011 + 0010 = 0101):-

Subtraction (1011 - 0101 = 1110):-

## CONCLUSION

A. Conclusion and Key Findings

The Binary Calculator Using Logic Gates project successfully demonstrates the practical application of Boolean algebra and digital electronics in modern computing. By implementing a 4-bit Ripple Carry Adder-Subtractor, the project illustrates how a single hardware circuit can be multiplexed to perform multiple arithmetic operations using the 2's Complement method.

The project highlights the following key technical insights:

- Hardware Efficiency: The strategic use of XOR gates for conditional inversion proves that addition and subtraction do not require separate circuits, mirroring the architectural efficiency of actual Arithmetic Logic Units (ALUs).

- Data Interpretation: The implementation underscores that binary data is inherently neutral; its meaning (Unsigned vs. Signed) depends entirely on the architectural interpretation of the Most Significant Bit (MSB).

- Error Diagnostics: The integration of Carry-out and Overflow flags provides a clear distinction between unsigned range exhaustion and signed arithmetic errors                      (V = C3 ⊕ C4).

- Cross-Platform Consistency: Developing the logic in both MATLAB and JavaScript verifies that the underlying mathematical algorithms are platform-independent, robust, and mathematically sound.

B. Learning Outcomes

Through the development of this simulator, the following competencies were mastered:

- Digital Logic Design: Expertise in using XOR gates as conditional inverters to streamline circuit complexity.

- Mathematical Rigor: Deep understanding of 2's complement arithmetic and its role in binary subtraction.

LINK OF WORKING PROJECT

Simulator Link:- https://circuitverse.org/simulator/embed/binary-calculator-b827cd11-a3fa-4821-b068-63154f7b040b